Appendix

Arduino Code for Physical Circuit

#include<SerialCommand.h>

#include<SoftwareSerial.h>

SerialCommand SCmd; // The demo SerialCommand object

#define arduinoLED 13 // Arduino LED on board

boolean DEBUG = false;

// Variables for Caliper Reading

int i;

int sign;

long value;

float measure;

int clockpin = 8;

int datapin = 9;

unsigned long tempmicros;

unsigned long revMillis;

//Variables for Stepper Motor control

int ENA=2;//connected to Arduino's port 2

int IN1=3;//connected to Arduino's port 3

int IN2=4;//connected to Arduino's port 4

int ENB=5;//connected to Arduino's port 5

int IN3=6;//connected to Arduino's port 6

int IN4=7;//connected to Arduino's port 7

// Variables for measuring process

int nbCycles = 1;

int nbSteps = 600; // number of steps for a full

revolution

int stepCount = 0; // will 0/600

int currentStep = 1; // will loop 1/2/3/4

int stepDirection = -1; // 1 : clockwise / -1 :

counter clockwise

void setup() {

//Caliper Reading

Serial.begin(9600);

pinMode(clockpin, INPUT);

pinMode(datapin, INPUT);

//Stepper Motor

pinMode(ENA,OUTPUT);

pinMode(ENB,OUTPUT);

pinMode(IN1,OUTPUT);

pinMode(IN2,OUTPUT);

pinMode(IN3,OUTPUT);

pinMode(IN4,OUTPUT);

digitalWrite(ENA,HIGH);//enable motorA

digitalWrite(ENB,HIGH);//enable motorB

// Setup callbacks for SerialCommand commands

SCmd.addCommand("PING",ping);

SCmd.addCommand("STEP",doStep); //Do one step

SCmd.addCommand("MEASURE",getMeasure); //Get measure

and print result

SCmd.addCommand("SET_DIRECTION",setDirection); //Set

the direction of rotation

SCmd.addDefaultHandler(unrecognized); //

Handler for command that isn't matched

}

void loop()

{

SCmd.readSerial(); // Process serial commands

}

void getMeasure(){

if(!DEBUG){

readCalliper();

}

else{ //DEBUG VERSION

measure = random(0, 10);

}

//Serial.print("MEASURE ");

Serial.println(measure,2);

}

void doStep() {

currentStep = currentStep + stepDirection;

if(currentStep > 4) currentStep = 1;

if(currentStep < 1) currentStep = 4;

switch (currentStep) {

case 1:

digitalWrite(IN1,LOW);

digitalWrite(IN2,HIGH);

digitalWrite(IN3,HIGH);

digitalWrite(IN4,LOW);

break;

case 2:

digitalWrite(IN1,LOW);

digitalWrite(IN2,HIGH);

digitalWrite(IN3,LOW);

digitalWrite(IN4,HIGH);

break;

case 3:

digitalWrite(IN1,HIGH);

digitalWrite(IN2,LOW);

digitalWrite(IN3,LOW);

digitalWrite(IN4,HIGH);

break;

case 4:

digitalWrite(IN1,HIGH);

digitalWrite(IN2,LOW);

digitalWrite(IN3,HIGH);

digitalWrite(IN4,LOW);

}

Serial.println("STEP_ACK");

}

void readCalliper() {

while (digitalRead(clockpin)==HIGH) {

} //if clock is LOW wait until it turns to HIGH

tempmicros=micros();

while (digitalRead(clockpin)==LOW) {

} //wait for the end of the HIGH pulse

if ((micros()-tempmicros)>500) { //if the HIGH pulse

was longer than 500 micros we are at the start of a

new bit sequence

decodeCalliper(); //decode the bit sequence

}

}

void decodeCalliper() {

sign=1;

value=0;

for (i=0;i<23;i++) {

while (digitalRead(clockpin)==HIGH) {

} //wait until clock returns to HIGH- the first

bit is not needed

while (digitalRead(clockpin)==LOW) {

} //wait until clock returns to LOW

if (digitalRead(datapin)==LOW) {

if (i<20) {

value|= 1<<i;

}

if (i==20) {

sign=-1;

}

}

}

measure=(value*sign)/100.00;

}

void ping()

{

Serial.println("PING_ACK");

}

void setDirection() {

String arg = SCmd.next();

if (arg != NULL) {

stepDirection = arg.toInt();

}

Serial.println("DIR_ACK");

}

// This gets set as the default handler, and gets

called when no other command matches.

void unrecognized()

{

Serial.println("No Value");

}